## 9V1A Adapter

FYWS-91 (SMPS Adaptor) is designed on Highly Integrated SMPS Switcher IC which makes this product reliable and highly efficient.

- 9V1A Rated Output.

- It comes with 1.5mtr wire length and 5.5*2.1mm DC Plug Size.

- Led indicator to show the functional status.

- Made from High grade ABS plastic

- Overload, Over Voltage & Over Current Protection.

- Short Circuit, Over Temperature Protection.

- Unique and attractive design.

- BIS Certified.

Categories: Power Adaptors, Wallmount Series

| Feature | Specification |
| --- | --- |
| VAC Voltage | 150~265VAC |
| Output voltage | 9VDC |
| Rated Power | 9W |
| Ripple & Noise (max.) | 70mV(max) |
| Voltage Adj. Range | 5V |
| Device | Primary-Side Regulation CV/CC Switcher for Adapters and Chargers |
| Device(frequency) | 85KHz |
| VDS Voltage | 600V(max) |
| Line Regulations | ±1% |
| Load Regulations | ±5% |
| SCP | Output short-circuit protection |
| ARP | Auto-restart protection reduces power delivered by >90% for output short-circuit and control loop faults (open and shorted components) |
| ON/OFF control | ON/OFF control provides constant efficiency down to very light loads – ideal for CEC regulations |
| line UV | Line brown-in/out protection |
| OV | Line overvoltage shutdown extends line surge withstand |
| OTP | Accurate thermal shutdown, hysteretic or latching |
| Efficiency(Typ.) | 80% |
| Frequency Range | 47~63HZ |
| Storage Temperature | 0-50°C |